# Report

QA İŞLEV GeoTek

# Project information

| Item | Value |
| --- | --- |
| Project name | QA İŞLEV GeoTek |
| Project no. | QA-FUN |
| Engineer | QA denetimi — tasarım değildir |
| Date | 2026-10-22 |
| Design code | AASHTO LRFD |
| Analysis level | Basic |
| PGA [g] | - |
| Mw | - |
| SDS | - |
| SD1 | - |
| Site class | - |

# Soil profile: QA Kum 0-40m (kaynak test)

| Item | Value |
| --- | --- |
| GWT in situ [m] | 100.00 |
| GWT design [m] | 100.00 |
| GWT seismic [m] | 100.00 |

| Top [m] | Bottom [m] | Description | USCS | γ [kN/m³] | γsat [kN/m³] | φ' [°] | c' [kPa] | su [kPa] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 40.00 | - | - | 18.00 | 20.00 | 30.0 | 0.0 | - |

# Soil profile: QA Sulu iki tabaka

| Item | Value |
| --- | --- |
| GWT in situ [m] | 2.00 |
| GWT design [m] | 2.00 |
| GWT seismic [m] | 2.00 |

| Top [m] | Bottom [m] | Description | USCS | γ [kN/m³] | γsat [kN/m³] | φ' [°] | c' [kPa] | su [kPa] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 6.00 | - | - | 18.00 | 20.00 | 30.0 | 0.0 | - |
| 6.00 | 40.00 | - | - | 19.00 | 21.00 | 32.0 | 0.0 | - |

# Retaining structure: QA Konsol 5m

| Item | Value |
| --- | --- |
| Wall type | Cantilever wall |
| Height [m] | 5.00 |
| Backfill slope [°] | 10.0 |
| Surcharge [kPa] | 15.00 |
| Surcharge basis | Horizontal projection |
| GWT case | Design |
| Design approach | - |
| kh | 0.133 |
| kv | 0.067 |

### Geometry

| Item | Value |
| --- | --- |
| Stem height | 4.550 |
| Stem top width | 0.300 |
| Stem bottom width | 0.450 |
| Toe | 2.000 |
| Heel | 3.550 |
| Base thickness | 0.450 |
| Embedment D | 0.600 |

### Shear key results

| Item | Value |
| --- | --- |
| Position | Under heel |
| Key width (m) | 0.50 |
| Key depth (m) | 0.50 |

| Case | FS base surface | FS key-tip plane | FS without key | FS with key | Passive used (kN/m) | Key shear demand V (kN/m) | Key moment demand M (kNm/m) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Static | 2.11 | 2.01 | 1.99 | 2.01 | 11.5 | 22.9 | 6.3 |
| Seismic | 1.15 | 1.11 | 1.10 | 1.11 | 9.8 | 22.9 | 6.3 |

Key depth is not added to the bearing embedment (conservative); no reinforced-concrete design of the key, only V and M demands.

Result: Pass

Governing check: Seismic · Sliding

### Checks

| Case | Check | Value | Required | Utilisation | Result |
| --- | --- | --- | --- | --- | --- |
| Static | Overturning | 6.704 | >= 2.000 | 0.30 | Pass |
| Static | Sliding | 2.008 | >= 1.500 | 0.75 | Pass |
| Static | Eccentricity (e/B) | 0.068 | <= 0.167 | 0.41 | Pass |
| Static | Bearing capacity | 5.665 | >= 3.000 | 0.53 | Pass |
| Seismic | Overturning | 3.386 | >= 1.500 | 0.44 | Pass |
| Seismic | Sliding | 1.106 | >= 1.100 | 0.99 | Pass |
| Seismic | Eccentricity (e/B) | 0.026 | <= 0.250 | 0.10 | Pass |
| Seismic | Bearing capacity | 2.330 | >= 2.000 | 0.86 | Pass |
| Static | Global stability | 1.602 | >= 1.500 | 0.94 | Pass |
| Seismic | Global stability | 1.180 | >= 1.100 | 0.93 | Pass |

### Assumptions

Beton birim hacim ağırlığı 24 kN/m³ (WallSettings; mevcut varsayılan 24 kN/m³, proje girdisi)

Statik YASS senaryosu design; arka derinlik 100 m, ön derinlik 100 m (StructureSoilModel; None olduğunda profil YASS'ı)

Taban, tutulan zemin yüzeyinden H derinliktedir; ön zemin H-D kotundadır; ön kazının kaldırdığı örtü gerilmesi hesaba katılmaz (P1 sözleşmesi §1 ve §5)

Katmanlar yataydır; her katmanın Coulomb katsayısı tüm üst katmanların düşey gerilmesine uygulanır; bu yaklaşım katmanlı bir kritik kama çözümü değildir (Das, Principles of Foundation Engineering; P1 §5 yerel katsayı yaklaşımı)

Coulomb drene itkide c' katkısı ihmal edilir; drenajsız yatay zeminde toplam gerilme ve s_u kullanılır, yatay su ikinci kez eklenmez (lateral_pressure sözleşmesi; Rankine toplam gerilme çözümü)

Topuk üzerindeki zemin katman ve su kotlarında poligonlara bölünür; su altında toplam γ_sat ağırlığı ve tabanda ayrıca kaldırma kullanılır (Statik denge; Terzaghi etkin gerilme ilkesi)

Arka/ön su seviyesi yapı YASS değeridir (yoksa profil YASS); hidrostatik yatay basınç ve ön/arka taban basınçları arasında doğrusal kaldırma; sızma hesabı yok (P1 §1.4; USACE EM 1110-2-2502 hidrostatik yük modeli)

Taşıma gücü yalnız taban katmanıyla hesaplanır; B içinde daha zayıf katman uyarılır; iki tabakalı taşıma gücü ve oturma hesabı yapılmaz (P1 §5)

Taban delta_base değeri verilmemişse mevcut FoundationSoil varsayılanıyla delta_base=phi kullanılır; aderans 0 alınır (FoundationSoil sözleşmesi)

Taşıma gücü yöntemi: vesic; eğik yük ve B'=B-2|e| dahil; taban/ön arazi eğimi çarpanları 1 (Mevcut bearing_capacity/profil adaptörü; Bowles, 5. baskı, Tablo 4-1/4-5)

Kaldırma momenti devrilme talebine eklenir: FS_ot=M_R/(M_O+M_U) (Mühendis seçimi; varsayılan güvenli taraf)

Drene N_gamma YASS düzeltmesi açık: d_w ön suyun taban altı derinliği; d_w≤0 için γ', 0<d_w<B için γ'+(d_w/B)(γ−γ'), d_w≥B için γ (Das, Principles of Foundation Engineering, taşıma gücüne yeraltı suyu etkisi (Durum III))

Gömülme D = 0.60 m; boyut araması 0.05 m adımla B ≤ 1.2H (Ön boyutlandırma oranları (Das, Principles of Foundation Engineering; mühendislik yargısı))

Boyutlandırmada kesit oranları mevcut proportion_for fonksiyonundan alınır (Ön boyutlandırma oranları (Das, Principles of Foundation Engineering; mühendislik yargısı))

ASD global GS durumunda yük katsayılanmaz; kısmi katsayıda WA ayrı kullanıcı girdisidir ve G/EH katsayısından türetilmez; material.gamma uygulanmaz, DC/EV yük katsayıları kullanılır (P1 §5; mevcut limit_state sözleşmesi)

Topuk ucundan düşey sanal düzlem: H_v=H+topuk·tanβ; ek üst kısım en üst katmandır; δ=β (Klasik konsol duvar yaklaşımı; P1 §5)

Pasif direnç hesaba katılmadı (Kullanıcı/WallSettings; varsayılan 0.5 için doğrulanmış normatif kaynak yok)

Shear key (heel, b = 0.5 m, d = 0.5 m): passive resistance on the front face with Rankine K_p; vertical stress starts at the front ground (front fill + foundation γ); reduction factor 0.5 (same as FS_p = 2 in the METU CE366 example); reduced by (1−k_v)K_PE/K_P in the seismic case. Key passive counts regardless of the toe-passive setting; if the front soil may be removed by excavation or scour, enter embedment D = 0 (Das, Principles of Foundation Engineering (taban dişi, D1 = D + diş derinliği); ODTÜ CE366 Recitation 5 P1 (P_p = 65D + 32.5D², FS_p = 2))

Sliding is checked on two surfaces: the base interface (V tanδ_b + c_a·B′ + key passive) and the horizontal plane through the key tip ((ΣV + soil weight beside the key) tanφ + c·B′ + key passive). On the key-tip plane the thrust on the heel line is extended to the key bottom (foundation K_a, δ = β; surcharge, water and seismic increments and the inertia of the soil beside the key included). The smaller ratio governs. With an undrained foundation, seismic loading or sloping backfill, the key-zone thrust cannot be computed; the analysis stops with an explicit error (Das, Principles of Foundation Engineering (taban dişi, D1 = D + diş derinliği); ODTÜ CE366 Recitation 5 P1 (P_p = 65D + 32.5D², FS_p = 2))

Key depth is not added to the bearing embedment (conservative in every case); only shear and moment demands are given for the key (V = unreduced static passive, M = V·arm); no reinforced-concrete design (Koordinatör kararı; tutucu)

Global stability is computed without the key: the Bishop circle passes below the base and the key volume is treated as soil. The strength side is conservative (concrete is stronger than soil); the neglected concrete-soil weight difference has a small effect (Koordinatör kararı; tutucu)

Ön ayak üzerindeki doğal zemin ağırlığı ihmal edildi (WallSettings)

Sürşarjın topuk üzerindeki düşey yükü dengeleyici sayılmadı (WallDesignInputs)

Deprem YASS senaryosu seismic; arka derinlik 100 m, ön derinlik 100 m (StructureSoilModel; depth_seismic, yoksa depth_insitu)

k_h=0.4·S_DS/r, |k_v|=0.5·k_h, r=1.5; dinamik itki artımı H_v/2'de (TBDY 2018 Denk. 16.22, Tablo 16.7, 16.12.2.8)

M-O katman bazında; statik su hidrostatiktir; sıvılaşma ve fazla boşluk suyu basıncı hesaplanmaz; drenajsız deprem itkisi durdurulur; β > φ' − ψ için TBDY Denk. 16.24b kullanılır (Mononobe & Matsuo (1929), Okabe (1926); TBDY Denk. 16.24b; E4 kapsam sınırı)

Bishop daireleri taban altından geçer; taşınan ağırlık taban izine eşdeğer yük olarak yayılır; kayma modelinde tek yatay su kotu için ön/arka YASS'ın yükseği alınır; bantlar yatay ve en alt bant sonsuzdur; ön taraftaki taban üstü ince şerit de tutulan taraf bantlarıyla temsil edilir; global dilimlerde k_v uygulanmaz (Bishop (1955); mevcut wall_global_stability yaklaşımı; E4 model varsayımı)

### Factors used

| Factor | Value | Source | Status |
| --- | --- | --- | --- |
| safety.overturning.static | 2.000 | Eksantrisite kriterinden türetilmiş yaklaşık değer; doğrudan kaynak yok | Unverified |
| safety.sliding.static | 1.500 | FHWA-NHI-06-089 Vol. II (2026-10-20 karşılaştırma, secondary) | Secondary source |
| eccentricity.soil.static | 0.167 | FHWA-NHI-06-089 Vol. II, B/6 (secondary) | Secondary source |
| safety.bearing_capacity.static | 3.000 | FHWA-NHI-06-089 Vol. II, önerilen 3.0 (secondary) | Secondary source |
| safety.overturning.seismic | 1.500 | ASD — yaygın uygulama değeri (FHWA-NHI-06-089 ile karşılaştırılmadı) | Unverified |
| safety.sliding.seismic | 1.100 | ASD — yaygın uygulama değeri (FHWA-NHI-06-089 ile karşılaştırılmadı) | Unverified |
| eccentricity.soil.seismic | 0.250 | ASD — yaygın uygulama; sismik değerler doğrulanmadı | Unverified |
| safety.bearing_capacity.seismic | 2.000 | ASD — yaygın uygulama değeri (FHWA-NHI-06-089 ile karşılaştırılmadı) | Unverified |
| safety.global_stability.static | 1.500 | FHWA-NHI-06-089 Vol. II (secondary) | Secondary source |
| safety.global_stability.seismic | 1.100 | ASD — yaygın uygulama değeri (FHWA-NHI-06-089 ile karşılaştırılmadı) | Unverified |

### Warnings

Key depth is not added to the bearing embedment (deliberate conservative assumption: bearing capacity is under-estimated)

Düşey deprem katsayısı ±kv zarfı uygulandı (TBDY 16.12.2.3); yöneten işaret kontrol bazında detayda belirtilmiştir.

# Sheet pile: QA Konsol palplanş

| Item | Value |
| --- | --- |
| Support | Cantilever |
| Backfill slope β [°] | 10.0 |
| Surcharge [kPa] | 15.00 |
| Surcharge basis | Horizontal projection |
| Design approach | - |
| Retained height H [m] | 4.00 |
| Theoretical embedment D0 [m] | 5.98 |
| Design embedment D [m] | 7.18 |
| Total length [m] | 11.18 |
| Anchor depth [m] | - |
| Anchor force [kN/m] | - |
| Toe force [kN/m] | 283.9 |
| Max moment [kNm/m] | 324.0 |
| Depth of max moment [m] | 7.40 |
| Required section modulus [cm³/m] | - |

### Assumptions

Layered Rankine active/passive pressure; separate front/retained profiles and groundwater; hydrostatic water; no seepage (Rankine (1857); P1 §1)

Passive pressure divided by FS=1.5; where smaller, the passive pressure from tan φ′ and c′ divided by FS is used (high φ′; USACE EM 1110-2-2504 5-2d) (USACE EM 1110-2-2504 Tablo 5-1: perde duvarı, serbest drenajlı zemin, olağan durum 1.50 (S durumu); ince daneli zeminde Q durumu 2.00 (programda ayrı değer yok))

Cantilever Blum embedment increased by 20%; free-earth support for anchored walls (Mevcut sheet_pile.py; USS Steel Sheet Piling Design Manual / USACE EM 1110-2-2504)

Sloping backfill β = 10.0°: Rankine slope coefficient (resultant parallel to the surface), front side horizontal; c′ ignored on sloping ground; seismic M-O (δ = β) stops when β > φ′ − ψ (Rankine (1857); Mononobe-Okabe; Das, Principles of Foundation Engineering Ch. 7)

# Sheet pile: QA Ankrajlı palplanş

| Item | Value |
| --- | --- |
| Support | Single anchor |
| Backfill slope β [°] | 10.0 |
| Surcharge [kPa] | 15.00 |
| Surcharge basis | Horizontal projection |
| Design approach | - |
| Retained height H [m] | 4.00 |
| Theoretical embedment D0 [m] | 2.44 |
| Design embedment D [m] | 2.44 |
| Total length [m] | 6.44 |
| Anchor depth [m] | 1.00 |
| Anchor force [kN/m] | 54.4 |
| Toe force [kN/m] | - |
| Max moment [kNm/m] | 60.1 |
| Depth of max moment [m] | 3.44 |
| Required section modulus [cm³/m] | - |

### Assumptions

Layered Rankine active/passive pressure; separate front/retained profiles and groundwater; hydrostatic water; no seepage (Rankine (1857); P1 §1)

Passive pressure divided by FS=1.5; where smaller, the passive pressure from tan φ′ and c′ divided by FS is used (high φ′; USACE EM 1110-2-2504 5-2d) (USACE EM 1110-2-2504 Tablo 5-1: perde duvarı, serbest drenajlı zemin, olağan durum 1.50 (S durumu); ince daneli zeminde Q durumu 2.00 (programda ayrı değer yok))

Cantilever Blum embedment increased by 20%; free-earth support for anchored walls (Mevcut sheet_pile.py; USS Steel Sheet Piling Design Manual / USACE EM 1110-2-2504)

Sloping backfill β = 10.0°: Rankine slope coefficient (resultant parallel to the surface), front side horizontal; c′ ignored on sloping ground; seismic M-O (δ = β) stops when β > φ′ − ψ (Rankine (1857); Mononobe-Okabe; Das, Principles of Foundation Engineering Ch. 7)

Anchor load = wall force × spacing / cos(inclination); spacing 1.0 m, inclination 0.0° (SheetPileStructure; geotek.engine.sheet_pile.anchored_sheet_pile)

# Retaining structure: QA Ağırlık sulu 5m

| Item | Value |
| --- | --- |
| Wall type | Gravity wall |
| Height [m] | 5.00 |
| Backfill slope [°] | 0.0 |
| Surcharge [kPa] | 15.00 |
| Surcharge basis | Horizontal projection |
| GWT case | Design |
| Design approach | - |
| kh | 0.133 |
| kv | 0.067 |

### Geometry

| Item | Value |
| --- | --- |
| Height H | 5.000 |
| Base width B | 3.000 |
| Top width | 0.800 |
| Back face set-back | 0.200 |
| Embedment D | 0.600 |

Result: Fail

Governing check: Static · Overturning; Static · Sliding; Static · Bearing capacity; Seismic · Overturning; Seismic · Sliding; Seismic · Eccentricity (e/B); Seismic · Bearing capacity; Static · Global stability; Seismic · Global stability

### Checks

| Case | Check | Value | Required | Utilisation | Result |
| --- | --- | --- | --- | --- | --- |
| Static | Overturning | 1.853 | >= 2.000 | 1.08 | Fail |
| Static | Sliding | 1.430 | >= 1.500 | 1.05 | Fail |
| Static | Eccentricity (e/B) | 0.074 | <= 0.167 | 0.45 | Pass |
| Static | Bearing capacity | 1.396 | >= 3.000 | 2.15 | Fail |
| Seismic | Overturning | 1.200 | >= 1.500 | 1.25 | Fail |
| Seismic | Sliding | 0.708 | >= 1.100 | 1.55 | Fail |
| Seismic | Eccentricity (e/B) | 0.341 | <= 0.250 | 1.36 | Fail |
| Seismic | Bearing capacity | 0.023 | >= 2.000 | 85.17 | Fail |
| Static | Global stability | 0.684 | >= 1.500 | 2.19 | Fail |
| Seismic | Global stability | 0.437 | >= 1.100 | 2.52 | Fail |

### Assumptions

Beton birim hacim ağırlığı 24 kN/m³ (WallSettings; mevcut varsayılan 24 kN/m³, proje girdisi)

Statik YASS senaryosu design; arka derinlik 2 m, ön derinlik 2 m (StructureSoilModel; None olduğunda profil YASS'ı)

Taban, tutulan zemin yüzeyinden H derinliktedir; ön zemin H-D kotundadır; ön kazının kaldırdığı örtü gerilmesi hesaba katılmaz (P1 sözleşmesi §1 ve §5)

Katmanlar yataydır; her katmanın Coulomb katsayısı tüm üst katmanların düşey gerilmesine uygulanır; bu yaklaşım katmanlı bir kritik kama çözümü değildir (Das, Principles of Foundation Engineering; P1 §5 yerel katsayı yaklaşımı)

Coulomb drene itkide c' katkısı ihmal edilir; drenajsız yatay zeminde toplam gerilme ve s_u kullanılır, yatay su ikinci kez eklenmez (lateral_pressure sözleşmesi; Rankine toplam gerilme çözümü)

Topuk üzerindeki zemin katman ve su kotlarında poligonlara bölünür; su altında toplam γ_sat ağırlığı ve tabanda ayrıca kaldırma kullanılır (Statik denge; Terzaghi etkin gerilme ilkesi)

Arka/ön su seviyesi yapı YASS değeridir (yoksa profil YASS); hidrostatik yatay basınç ve ön/arka taban basınçları arasında doğrusal kaldırma; sızma hesabı yok (P1 §1.4; USACE EM 1110-2-2502 hidrostatik yük modeli)

Taşıma gücü yalnız taban katmanıyla hesaplanır; B içinde daha zayıf katman uyarılır; iki tabakalı taşıma gücü ve oturma hesabı yapılmaz (P1 §5)

Taban delta_base değeri verilmemişse mevcut FoundationSoil varsayılanıyla delta_base=phi kullanılır; aderans 0 alınır (FoundationSoil sözleşmesi)

Taşıma gücü yöntemi: vesic; eğik yük ve B'=B-2|e| dahil; taban/ön arazi eğimi çarpanları 1 (Mevcut bearing_capacity/profil adaptörü; Bowles, 5. baskı, Tablo 4-1/4-5)

Kaldırma momenti devrilme talebine eklenir: FS_ot=M_R/(M_O+M_U) (Mühendis seçimi; varsayılan güvenli taraf)

Drene N_gamma YASS düzeltmesi açık: d_w ön suyun taban altı derinliği; d_w≤0 için γ', 0<d_w<B için γ'+(d_w/B)(γ−γ'), d_w≥B için γ (Das, Principles of Foundation Engineering, taşıma gücüne yeraltı suyu etkisi (Durum III))

Gömülme D = 0.60 m; boyut araması 0.05 m adımla B ≤ 1.2H (Ön boyutlandırma oranları (Das, Principles of Foundation Engineering; mühendislik yargısı))

Boyutlandırmada kesit oranları mevcut proportion_for fonksiyonundan alınır (Ön boyutlandırma oranları (Das, Principles of Foundation Engineering; mühendislik yargısı))

ASD global GS durumunda yük katsayılanmaz; kısmi katsayıda WA ayrı kullanıcı girdisidir ve G/EH katsayısından türetilmez; material.gamma uygulanmaz, DC/EV yük katsayıları kullanılır (P1 §5; mevcut limit_state sözleşmesi)

İtki gerçek eğimli arka yüzde; θ=atan(arka kaçıklık/H), katman başına δ=2φ'/3 (TBDY 2018 16.12.2.9 (aktif durumda δ ≤ 2φ'/3))

Pasif direnç hesaba katılmadı (Kullanıcı/WallSettings; varsayılan 0.5 için doğrulanmış normatif kaynak yok)

Ön ayak üzerindeki doğal zemin ağırlığı ihmal edildi (WallSettings)

Sürşarjın topuk üzerindeki düşey yükü dengeleyici sayılmadı (WallDesignInputs)

Deprem YASS senaryosu seismic; arka derinlik 2 m, ön derinlik 2 m (StructureSoilModel; depth_seismic, yoksa depth_insitu)

k_h=0.4·S_DS/r, |k_v|=0.5·k_h, r=1.5; dinamik itki artımı H_v/2'de (TBDY 2018 Denk. 16.22, Tablo 16.7, 16.12.2.8)

M-O katman bazında; statik su hidrostatiktir; sıvılaşma ve fazla boşluk suyu basıncı hesaplanmaz; drenajsız deprem itkisi durdurulur; β > φ' − ψ için TBDY Denk. 16.24b kullanılır (Mononobe & Matsuo (1929), Okabe (1926); TBDY Denk. 16.24b; E4 kapsam sınırı)

Su altı deprem seçimi: envelope; γ*=γ_sat−γ_w; geçirimsiz tanψ=γ_sat/γ*·kh/(1−kv), geçirimli tanψ=γ/γ*·kh/(1−kv) (γ doğal birim ağırlık); dinamik su kh=0.4·S_DS (r=1). Taban sanal arka düzlemin altıdır; kısmi batıklık katman bazında mühendislik yargısıdır; karışık geçirgenlikte herhangi bir geçirimli katman için tam su yüksekliği bir kez kullanılır (doğrulanmamış mühendislik yaklaşımı); dış serbest su modellenmez. Yöneten seçim durum uyarılarında; dinamik suyun kuvvet ve konumu forces içindedir. (TBDY 2018 16.12.3.2, Denk. 16.26 ve 16.20)

Bishop daireleri taban altından geçer; taşınan ağırlık taban izine eşdeğer yük olarak yayılır; kayma modelinde tek yatay su kotu için ön/arka YASS'ın yükseği alınır; bantlar yatay ve en alt bant sonsuzdur; ön taraftaki taban üstü ince şerit de tutulan taraf bantlarıyla temsil edilir; global dilimlerde k_v uygulanmaz (Bishop (1955); mevcut wall_global_stability yaklaşımı; E4 model varsayımı)

### Factors used

| Factor | Value | Source | Status |
| --- | --- | --- | --- |
| safety.overturning.static | 2.000 | Eksantrisite kriterinden türetilmiş yaklaşık değer; doğrudan kaynak yok | Unverified |
| safety.sliding.static | 1.500 | FHWA-NHI-06-089 Vol. II (2026-10-20 karşılaştırma, secondary) | Secondary source |
| eccentricity.soil.static | 0.167 | FHWA-NHI-06-089 Vol. II, B/6 (secondary) | Secondary source |
| safety.bearing_capacity.static | 3.000 | FHWA-NHI-06-089 Vol. II, önerilen 3.0 (secondary) | Secondary source |
| safety.overturning.seismic | 1.500 | ASD — yaygın uygulama değeri (FHWA-NHI-06-089 ile karşılaştırılmadı) | Unverified |
| safety.sliding.seismic | 1.100 | ASD — yaygın uygulama değeri (FHWA-NHI-06-089 ile karşılaştırılmadı) | Unverified |
| eccentricity.soil.seismic | 0.250 | ASD — yaygın uygulama; sismik değerler doğrulanmadı | Unverified |
| safety.bearing_capacity.seismic | 2.000 | ASD — yaygın uygulama değeri (FHWA-NHI-06-089 ile karşılaştırılmadı) | Unverified |
| safety.global_stability.static | 1.500 | FHWA-NHI-06-089 Vol. II (secondary) | Secondary source |
| safety.global_stability.seismic | 1.100 | ASD — yaygın uygulama değeri (FHWA-NHI-06-089 ile karşılaştırılmadı) | Unverified |

### Warnings

Eğimli arka yüzdeki zemin ağırlığı Coulomb itkisine dahildir; ayrıca eklenmedi

Kazı Destek Yapıları Tasarım ve Uygulama Esasları (16.07.2026 değişikliği) ek şartları kontrol edilmelidir; bu belge yazılımda incelenmedi.

Kısmen su altındaki dolgu yönetmelik formüllerinin kapsamı dışındadır; katman bazında mühendislik yargısı uygulandı — sonucu kontrol edin.

Su altındaki dolgu: deprem itkisi TBDY 2018 Denk. 16.26 ile hesaplandı (EN 1998-5 Ek E ile uyumlu); seçim: envelope, yöneten: pervious.

Düşey deprem katsayısı ±kv zarfı uygulandı (TBDY 16.12.2.3); yöneten işaret kontrol bazında detayda belirtilmiştir.

coefficients.py notu: TBDY 2018 16.12.2.2 — boşluk suyu basıncı artışı olabilecek durumlarda r ≤ 1 gerekebilir; r değerini kontrol edin.

Su altındaki dolgu: TBDY Denk. 16.20 dinamik su ve 16.26 r=1 varsayar (water_kh=0.4·S_DS); r>1 ile zemin itkisinin azaltılması mühendis tarafından gerekçelendirilmelidir.

# Soil classification

## Robertson SBTn (CPT)

QA-CPT

| Top [m] | Bottom [m] | Zone | Name | Mean Ic |
| --- | --- | --- | --- | --- |
| 0.10 | 1.40 | 6 | Sands - clean sand to silty sand | 1.91 |
| 1.40 | 1.45 | 5 | Sand mixtures - silty sand to sandy silt | 2.06 |
| 1.45 | 3.65 | 6 | Sands - clean sand to silty sand | 1.95 |
| 3.65 | 3.70 | 5 | Sand mixtures - silty sand to sandy silt | 2.08 |
| 3.70 | 3.90 | 6 | Sands - clean sand to silty sand | 2.02 |
| 3.90 | 4.00 | 5 | Sand mixtures - silty sand to sandy silt | 2.06 |
| 4.00 | 4.10 | 6 | Sands - clean sand to silty sand | 2.04 |
| 4.10 | 4.15 | 5 | Sand mixtures - silty sand to sandy silt | 2.06 |
| 4.15 | 4.35 | 6 | Sands - clean sand to silty sand | 2.01 |
| 4.35 | 4.40 | 5 | Sand mixtures - silty sand to sandy silt | 2.07 |
| 4.40 | 4.45 | 6 | Sands - clean sand to silty sand | 2.05 |
| 4.45 | 4.50 | 5 | Sand mixtures - silty sand to sandy silt | 2.09 |
| 4.50 | 4.55 | 6 | Sands - clean sand to silty sand | 2.01 |
| 4.55 | 4.60 | 5 | Sand mixtures - silty sand to sandy silt | 2.06 |
| 4.60 | 4.65 | 6 | Sands - clean sand to silty sand | 2.02 |
| 4.65 | 4.75 | 5 | Sand mixtures - silty sand to sandy silt | 2.08 |
| 4.75 | 4.80 | 6 | Sands - clean sand to silty sand | 2.04 |
| 4.80 | 4.85 | 5 | Sand mixtures - silty sand to sandy silt | 2.12 |
| 4.85 | 4.90 | 6 | Sands - clean sand to silty sand | 2.01 |
| 4.90 | 5.00 | 5 | Sand mixtures - silty sand to sandy silt | 2.10 |
| 5.00 | 5.05 | 6 | Sands - clean sand to silty sand | 2.05 |
| 5.05 | 5.35 | 5 | Sand mixtures - silty sand to sandy silt | 2.10 |
| 5.35 | 5.40 | 6 | Sands - clean sand to silty sand | 2.04 |
| 5.40 | 5.75 | 5 | Sand mixtures - silty sand to sandy silt | 2.09 |
| 5.75 | 5.80 | 6 | Sands - clean sand to silty sand | 2.05 |
| 5.80 | 6.00 | 5 | Sand mixtures - silty sand to sandy silt | 2.12 |
| 6.00 | 9.00 | 3 | Clays - silty clay to clay | 3.36 |
| 9.00 | 15.05 | 6 | Sands - clean sand to silty sand | 1.48 |

# Liquefaction analysis

| Item | Value |
| --- | --- |
| Soil profile | QA Kum 0-40m (kaynak test) |
| Method | Youd et al. (2001) |
| amax [g] | 0.400 |
| Mw | 7.5 |
| Target FS | 1.10 |
| Seismic GWT [m] | 3.00 |
| Default FC [%] | 5.0 |

| Sounding | Minimum FS | Depth [m] | Liquefiable points | Marginal points |
| --- | --- | --- | --- | --- |
| QA-SPT | 0.16 | 7.50 | 7 | 0 |
| QA-CPT | 0.34 | 5.85 | 60 | 0 |

### Warnings

QA-SPT: Default FC = 5.0% used at depth(s) 1.5, 3, 4.5, 6, 7.5, 9, 10.5, 12, 13.5 m (no laboratory FC).

QA-SPT: 1 SPT record(s) at refusal; N is a lower bound at depth(s) 13.5 m.

QA-SPT: σ′v0 > 200 kPa: the Liao & Whitman C_N is outside its recommended range.

QA-SPT: Refusal blow counts are lower bounds; their FS is conservative.

# Anchored wall analysis

| Item | Value |
| --- | --- |
| Wall type | Sheet pile |
| Excavation height H (m) | 10.00 |
| Soil behaviour | Drained (φ′) |
| Retained ground slope (°) | 0.0 |
| Surcharge (kPa) | 0.00 |
| Surcharge basis | Per horizontal projection |
| Passive factor of safety | 1.50 |
| Embedment increase | — |
| Allowable stress (MPa) | — |
| Method | Apparent pressure, hinge method |
| Theoretical embedment D0 (m) | 6.80 |
| Design embedment D (m) | 8.16 |
| Wall length (m) | 18.16 |
| Base reaction R_B (kN/m) | 27.1 |
| Max moment (kNm/m) | 274.0 |
| Max shear (kN/m) | 87.1 |
| Required section modulus (cm³) (cm³/m) | — |

### Soil layers used

| Top [m] | Bottom [m] | γ [kN/m³] | φ' [°] | c' [kPa] | su [kPa] |
| --- | --- | --- | --- | --- | --- |
| 0.00 | 40.00 | 18.00 | 30.0 | 0.0 | — |

Apparent pressure envelope (sand): p_max = 45.0 kPa, total thrust 390.0 kN/m.

### Anchor levels

| Depth (m) | Inclination (°) | Spacing (m) | Horizontal load (kN/m) | Design load T (kN) | Unbonded length (m) | Bond length (m) | Tendon |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.50 | 0.0 | 2.50 | 120.6 | 301.6 | — | — | — |
| 4.50 | 0.0 | 2.50 | 126.9 | 317.2 | — | — | — |
| 7.50 | 0.0 | 2.50 | 115.4 | 288.5 | — | — | — |

### Assumptions

Apparent pressure envelope (sand), level ground: FHWA GEC-4 5.2.4–5.2.6 (sand and stiff clay trapezoid, figures 24 and 27; soft clay figure 23c). The method is empirical.

Anchor loads by the hinge method: hinges at every anchor except the first; bending moments from the same model.

Embedment: moment equilibrium about the toe under the base reaction R_B and the net pressure below the base (passive divided by the given factor of safety; where smaller, the passive pressure from tan φ′ and c′ divided by that factor is used, USACE EM 1110-2-2504 5-2d). This is deeper than the GEC-4 5.5.3 force criterion.

Embedment increase not given: D = D0 (1 + 0.2).

Anchor design inputs not given: anchor loads only, no free or bond length.

No steel or concrete section design: shear and moment demands are reported, M/W is compared with a given stress only.

### Warnings

External (global) stability and deep-seated (Kranz) failure are not checked. Analyse slip surfaces passing behind and through the anchors separately by limit equilibrium; GEC-4 5.8.3 requires FS ≥ 1.3 (1.5 for critical permanent works).

Embedment comes from moment equilibrium about the toe (deeper than the GEC-4 5.5.3 force criterion, on the safe side). A toe force residual of 257.9 kN/m is not balanced and is not included in the shear maximum; assess it separately for the embedded section.

### Sources

FHWA GEC-4 (FHWA-IF-99-015) 5.2.4-5.2.6, figures 23, 24, 27-29, eq. 9-12 (apparent earth pressure envelopes)

FHWA GEC-4 (FHWA-IF-99-015) 5.3.3, figures 33-34 (hinge and tributary methods), 5.4.1 (wall moments)

FHWA GEC-4 (FHWA-IF-99-015) 5.5.3 (embedment; passive factor of safety)

FHWA GEC-4 (FHWA-IF-99-015) 5.8.3 (external stability: not analysed, check separately)

# Ground anchor design

| Item | Value |
| --- | --- |
| Excavation height H (m) | 10.00 |
| Anchor head depth (m) | 6.25 |
| Anchor inclination α (°) | 15.0 |
| Soil friction angle φ′ (°) | 33.0 |
| Horizontal load per metre | 172.0 |
| Anchor spacing | 2.50 |
| Factor of safety FS (bond) | 2.00 |
| Load transfer used (kN/m) | 100.0 |
| Lock-off load ratio | 1.00 |
| Test load factor | 1.33 |
| Design load limit (× SMTS) | 0.60 |
| Lock-off limit (× SMTS) | 0.70 |
| Test load limit (× SMTS) | 0.80 |
| Tendon type | Strand |
| Anchor ground | Soil |
| Bond start behind wedge (m) | — |
| Source of the bond value | QA: tests/api/test_anchored_wall_api.py BODY |
| Design load T (kN) | 445.2 |
| Vertical component T_v (kN) | 115.2 |
| Lock-off load (kN) | 445.2 |
| Maximum test load (kN) | 592.1 |
| Unbonded length (m) | 4.50 |
| Bond length (m) | 8.90 |
| Total length (m) | 13.40 |
| Factor of safety FS (bond) | 2.00 |
| Load transfer used (kN/m) | 100.0 |
| Tendon | 3 × 15 mm strand, ASTM A416 Gr 270 (metric 1860) (FHWA GEC-4 (FHWA-IF-99-015) Table 10, p. 78 (1 strand)) |
| Tendon capacity SMTS (kN) | 782.1 |
| Design load / SMTS | 0.57 |

| Check | Demand | Limit | Result |
| --- | --- | --- | --- |
| Design load ≤ tendon limit | 445.17 | 469.26 | Pass |
| Lock-off load ≤ tendon limit | 445.17 | 547.47 | Pass |
| Test load ≤ tendon limit | 592.07 | 625.68 | Pass |
| Minimum cover over the bond zone centre | 4.50 | 8.57 | Pass |

### Assumptions

The bond zone starts 2.0 m beyond the failure surface, measured normal to the surface (GEC-4 figure 37a: χ = 1.5 m or 0.2H, whichever is greater).

Minimum unbonded length 4.5 m assumed (strand) (GEC-4 5.3.4).

The failure surface starts at the excavation base corner (GEC-4 5.3.2).

Level retained ground and a vertical wall face are assumed; sloping ground is not supported yet.

Maximum test load = 1.33 × design load (GEC-4 pp. 75-77).

Tendon limits 0.6 / 0.7 / 0.8 SMTS (design / lock-off / test), GEC-4 p. 77.

Load transfer value source: QA: tests/api/test_anchored_wall_api.py BODY (preliminary design; verify by anchor tests).

Minimum bond length 4.5 m (soil, strand) applied (GEC-4 appendix E, 3B).

### Sources

FHWA GEC-4 (FHWA-IF-99-015) sections 5.3.2-5.3.4, 5.3.6-5.3.8, fig. 37 (lengths, spacing, tendon limits)

FHWA GEC-4 (FHWA-IF-99-015) appendix E part 3B, p. E-15 (minimum bond length)

FHWA GEC-4 (FHWA-IF-99-015) Eq. 19-21, p. 66-67 (anchor load)